Dynamic Logic

Table of contents

Finding Sizing for 5-stage Dynamic Logic	2

Matlab Code	2

Matlab Result	3

Sweeping Transistor – NAND Gate Test	4

HSPICE Code	4

Awave Result	4

Sweeping Transistor – NAND Gate with subcircuit	5

Sweep_transistor.sp	5

Sweep_transistor.sp	5

Gate.sp	6

Awave Result	7

Paramters	8

Sweeping Static Logic Transistors	8

Sweep Static Logic HSPICE	8

Awave Code	10

Dynamic Logic Gates	11

5-Stage Dynamic Logic that Does Not Work	12

HSPICE	12

Awaves	13

6-Stage Dynamic Logic	14

HSPICE	14

Awaves	16

# Finding Sizing for 5-stage Dynamic Logic

Dynamic Logic chain with 3fF-not-nand-not-nand-not-cload. Try to find the sizing for HSPICE simulation.

## Matlab Code

% Dynamic Logic

% 3fF-NOT-NAND-NOT-NAND-NOT-Cload

%Parameters

g1=1; g2=5/3; g3=1; g4=5/3; g5=1;

b1=1; b2=4; b3=1; b4=8; b5=1;

nstage=5;

%find cload

cw=5.5; %from HSPICE extraction

cox=6; co=0.31; w=0.36; l=0.24;

cg = cox*w*l + 2*co*w;

cload = cw + 2*cg*32

%Find h

G=g1*g2*g3*g4*g5; B=b1*b2*b3*b4*b5; F=(cload)/(3*cg);

H=G*B*F

h=H^(1/nstage)

%Find f

f(1)=h/(b1*g1);

f(2)=h/(b2*g2);

f(3)=h/(b3*g3);

f(4)=h/(b4*g4);

f(5)=h/(b5*g5);

s(5)=(cload)/f(5);  s(4)=s(5)/f(4);

s(3)=s(4)/f(3);  s(2)=s(3)/f(2);

s(1)=s(2)/f(1)

% add cw1

g1=1; g2=1; g3=5/3; g4=1;

b1=1; b2=4; b3=1; b4=1;

nstage2=4;

cw1=1.8641; %from HSPICE

cload = s(5)*8+cw1;

F2 = (cload)/(3*cg);

%cw1 is M4 of the predecoder = 1.8641

B2= b1*b2*b3*b4;

G2 = g1*g2*g3*g4;

H2 = F2* G2* B2

h2 = H2^(1/nstage2)

f1(1)=h2/(b1*g1);

f1(2)=h2/(b2*g2);

f1(3)=h2/(b3*g3);

f1(4)=h2/(b4*g4);

s1(4)=(cload)/f1(4);

s1(3)=s1(4)/f1(3);

s1(2)=s1(3)/f1(2);

s1(1)=s1(2)/f1(1);

%get w

cox=6; co=0.31; w=0.36; l=0.24;

cg = cox*w*l + 2*co*w;

inv_min = 3*cg;

nand3_min = 5*cg;

w(5)=s(5)/nand3_min;

w(4)=s1(4)/inv_min;

w(3)=s1(3)/nand3_min;

w(2)=s1(2)/inv_min;

w(1)=s1(1)/inv_min

## Matlab Result

cload =   52.9624

H =  2.1160e+003

h =    4.6249

s =  Columns 1 through 2

2.2248   10.2895

Columns 3 through 4

7.1383   33.0139

Column 5

11.4515

H2 =  280.1037

h2 =

4.0910

w =

Columns 1 through 2

1.0000    4.0910

Columns 3 through 4

2.5104   10.2702

Column 5

3.0883

# Sweeping Transistor – NAND Gate Test

Trying to sweep logic gate sizes in HSPICE.

## HSPICE Code

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

.subckt NAND3 Vdd Gnd VinA VinB VinC Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w='wpmin'

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w='wpmin'

Mp3 Vout VinC Vdd Vdd pmos l=0.24u w='wpmin'

Mn1 Vmid1 VinA Gnd Gnd nmos l=0.24u w='wnmin*3'

Mn2 Vmid2 VinB Vmid1 Gnd nmos l=0.24u w='wnmin*3'

Mn3 Vout VinC Vmid2 Gnd nmos l=0.24u w='wnmin*3'

.ends

xnand vdd 0 vin1 vdd vdd vout  NAND3 M=multiplier

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.05ns 10ns sweep multiplier 1 5 1

.end

## Awave Result

# Sweeping Transistor – NAND Gate with subcircuit

Sweep one NAND gate with subcircuit in another file.

## Sweep_transistor.sp

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc 'gate.sp'

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xnand vdd 0 vin1 vdd vdd vout  NAND3 M='multiplier'

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.05ns 10ns sweep multiplier 1 5 1

.end

## Sweep_transistor.sp

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc 'gate.sp'

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xnand vdd 0 vin1 vdd vdd vout  NAND3 M='multiplier'

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.05ns 10ns sweep multiplier 1 5 1

.end

## Gate.sp

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.subckt INV vin vout vdd vss

M1 vout vin vss vss  nmos l=0.24u w='wnmin'

M2 vout vin vdd vdd  pmos l=0.24u w='wpmin'

.ends

.subckt NAND3 Vdd Gnd VinA VinB VinC Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w='wpmin'

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w='wpmin'

Mp3 Vout VinC Vdd Vdd pmos l=0.24u w='wpmin'

Mn1 Vmid1 VinA Gnd Gnd nmos l=0.24u w='wnmin*3'

Mn2 Vmid2 VinB Vmid1 Gnd nmos l=0.24u w='wnmin*3'

Mn3 Vout VinC Vmid2 Gnd nmos l=0.24u w='wnmin*3'

.ends

.subckt Sram Vdd Gnd vword vbit vbitbar

*M<name> <drain> <gate> <source> <bulk> <model> <geometry>

M0 2 5 Vdd Vdd pmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 Vdd 2 5 Vdd pmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 2 5 0 0 nmos L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 0 2 5 0 nmos L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 vbit vword 2 0 nmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 5 vword vbitbar 0 nmos L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

.ends

.subckt NOR Vdd Gnd VinA VinB Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w=0.6u

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w=0.6u

Mn1 Vout VinA vmid Gnd nmos l=0.24u w=0.72u

Mn2 vmid VinB Gnd Gnd nmos l=0.24u w=0.72u

.ends

## Awave Result

# Paramters

Cw = 12.60438 fF

Cw1 = 1.8641f

SRAM pitch = 1.92um

PMOS-NMOS ratio: 1.6 (Rabaey, 209)

# Sweeping Static Logic Transistors

The goal is the find the optimal delay by sweeping the width of transistors used in the decoder circuit.

## Sweep Static Logic HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'logic.sp'

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xnand1_b1 vdd 0 vdd vdd vin2 vin3 NAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin2 x NAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin2 y NAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin2 z NAND3 M='multi_nand1'

xinv2 vin3 vin4 vdd 0 INV M='multi_inv2'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vdd vdd vin4 vin5 NAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a NAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b NAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c NAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d NAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e NAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f NAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g NAND3 M='multi_nand3'

xinv3 vin5 vout vdd 0 INV M='multi_inv3'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram

xsram15 vdd 0 vout vbit15 vbitbar15 Sram

xsram16 vdd 0 vout vbit16 vbitbar16 Sram

xsram17 vdd 0 vout vbit17 vbitbar17 Sram

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram

xsram27 vdd 0 vout vbit27 vbitbar27 Sram

xsram28 vdd 0 vout vbit28 vbitbar28 Sram

xsram29 vdd 0 vout vbit29 vbitbar29 Sram

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.01ns 11ns sweep multi_nand3 9.27 11.27 1 sweep multi_nand1 3.09 5.09 1 sweep  multi_inv3 3.09 5.09 1

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

## Awave Code

The blue line is the static decoder. Not swept.

The multiplier swept.

# Dynamic Logic Gates

2-input-NAND with inverter [Nikolic, Lecture 19, page 12]

3fF-NOT-NAND-NOT-NAND-NOT-Cload dynamic circuit

## 5-Stage Dynamic Logic that Does Not Work

The problem with 5-stages is when all inputs are on, the output oscillate between vdd and zero. This is not suitable for a decoder.

### HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'gate.sp'

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 0.1n 50ps 5ns 10ns)

Vclk clk 0 pulse(0v 2.5v 0.1n 0.1n 50ps 2.5ns 5ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xnand1_b1 vdd 0 vin2 vin2 vin2 vin3 clk vin4 DNAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin2 x clk xx  DNAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin2 y clk yy DNAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin2 z clk zz  DNAND3 M='multi_nand1'

xinv2 vin3 vin4 vdd 0 INV M='multi_inv2'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vin4 vin4 vin4 vin5 clk vout DNAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a clk aa DNAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b clk bb DNAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c clk cc DNAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d clk dd DNAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e clk ee DNAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f clk ff DNAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g clk gg DNAND3 M='multi_nand3'

xinv3 vin5 vout vdd 0 INV M='multi_inv3'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram

xsram15 vdd 0 vout vbit15 vbitbar15 Sram

xsram16 vdd 0 vout vbit16 vbitbar16 Sram

xsram17 vdd 0 vout vbit17 vbitbar17 Sram

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram

xsram27 vdd 0 vout vbit27 vbitbar27 Sram

xsram28 vdd 0 vout vbit28 vbitbar28 Sram

xsram29 vdd 0 vout vbit29 vbitbar29 Sram

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.1ns 16ns *sweep multi_nand3

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

### Awaves

## 6-Stage Dynamic Logic

6 Stage Dynamic Logic will have the output oscillating during a read. However, for a decoder design, we need to have the output stay high for multiple cycles.

### HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'gate.sp'

*****************************************

* Parameter

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

.param multi_inv4 = 4.6

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 0.1n 50ps 5ns 10ns)

Vclk clk 0 pulse(0v 2.5v 0.1n 0.1n 50ps 2.5ns 5ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xinv2 vin2 vin3 vdd 0 INV M='multi_inv2'

xnand1_b1 vdd 0 vin3 vin3 vin3 vin4 clk vin5 DNAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin3 x clk xx  DNAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin3 y clk yy DNAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin3 z clk zz  DNAND3 M='multi_nand1'

xinv3 vin4 vin5 vdd 0 INV M='multi_inv3'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vin5 vin5 vin5 vin6 clk vout DNAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a clk aa DNAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b clk bb DNAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c clk cc DNAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d clk dd DNAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e clk ee DNAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f clk ff DNAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g clk gg DNAND3 M='multi_nand3'

xinv4 vin6 vout vdd 0 INV M='multi_inv4'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram

xsram15 vdd 0 vout vbit15 vbitbar15 Sram

xsram16 vdd 0 vout vbit16 vbitbar16 Sram

xsram17 vdd 0 vout vbit17 vbitbar17 Sram

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram

xsram27 vdd 0 vout vbit27 vbitbar27 Sram

xsram28 vdd 0 vout vbit28 vbitbar28 Sram

xsram29 vdd 0 vout vbit29 vbitbar29 Sram

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.1ns 16ns *sweep multi_nand3

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

### Awaves

6-Stage not-not-nand-not-nand-not circuit Transcient Plot

Conclusion. Dynamic is not suitable for the decoder we need.