EECS40                                                                               Summer 2006

University of California Berkeley

Midterm Exam # 2

August 8th, 2006

Instructor: Octavian Florescu

Name:  _____Hung________________________, _______Bill___________________

Last					First

Student ID: _________________________, Signature___________________________

- This is a closed book exam. However you may use one page of handwritten double sided notes.

- Non programmable calculators are allowed only.

- Make sure to write down your name on every page.

| Problem | Mark |
| --- | --- |
| 1 | /25 |
| 2 | /25 |
| 3 | /20 |
| 4 | 30/30 |
| Total | /100 |

Name_ _ Good Student _

Problem 4.  MOSFET [30 points]

R1

R2

RS

VG

VDD

M1

vOUT

IBIAS

iIN

CGS

I1

Parameters:

VDD = 9V

IBIAS = 2mA

R1 = 3kΩ

R2 = 6kΩ

RS = 500Ω

VT(M1) = 0.5V

λ(M1) = 0.5V-1

μnCox(M1) = 100 uA/V2

W/L(M1) = 2

- The voltage VG considering IIN = 1mA. Note that iIN = iin + IIN [5pts]

VG=VDD−I1⋅R1I1=iIN+VGR2VG=9V−(iIN+VG6k)⋅3k32VG=9V−iIN⋅3kΩ32VG=9V−(iin+1mA)⋅3kΩVG=4V−2k⋅iin

VG = 4V if iin is 0 for DC analysis

Mistake 1: Ignoring IIN = 1mA, and just do a voltage divide. (-0.5 point)

Mistake 2: Plug in the wrong values for unknown reasons. (-0.5 point)

Mistake 3: Wrong sign when setting up the equation(+ or -) (-0.5 point)

- Find the source voltage of M1. [5pts]

VS=IBIAS⋅RS=1V

- Find VDS using the values from a. and b. What region is M1 operating in? [5pts]

Assume M1 is in saturation region.

IDS=μCoxW2L(VGS−Vt)2(1+λVDS)2mA=(100μAV2)22(3V−0.5V)2(1+0.5V−1⋅VDS)VDS=4.4V,VGS=3V,Vt=0.5V

Mistake 1:Wrong Vds (-0.5)

Mistake 2:No Vds but otherwise correct (-1)

VDS>VGS−Vt and VGS>Vt

Assumption was correct, and M1 is in Saturation Region.

- Find the transfer function vg / iin. Note that iin is a small signal without a DC component. [5pts]

Vg = -iin (R1//R2// Z(CGS) )

Vgiin=−1(1R1+1R2+jwCGS)

Common Mistake 1:Forgetting the negative sign (-0.5 point)

Common Mistake 2: Inverting the answer Vg/iin because the student forgot how to calculate impedance in parallel (-0.5 point)

Common Mistake 3: Both Mistake 1 and Mistake 2. (-1 point)

- Sketch the small signal model of this configuration and find the transfer function vout / iin. [10 pts: 5pts for the small signal model, 5 pts for the calculations]

Step 1. Replace the circuit with all small circuit elements. That is eliminate all the DC (constant voltage or constant current) elements.

R1

R2

RS

VG

vOUT

iin

CGS

gmVGS

ro

VS

Step 2. which gives a small signal model of:

R2

iin

R1

RS

Vout

gmVgs

ro

+

-

CGS

Vgs

+

-

- Notice, the small signal circuit is a new circuit, the “big” signal VG, VS, and VOUT are not here. Only small signal elements exist in a small signal model.

- CGS is a misnomer. It is not the capacitor between the gate and source of M1.

RS

Vout

gmVgs

ro

+

-

I1

I2

I1 = gmVgs

I1=Vs−VoutroVout=(I2−I1)ro+I2⋅RS

I2=VsRs

gm=μnCox(WL)(VGSQ−Vt)=0.5mro=1λIDS=1k

I1 = 0.5m (Vg – Vs)

I1=Vs−Vout1k

vout=(I2−I1)⋅1k+I2⋅500

I2=Vs500

Vout = - 1.375 Vg

Voutiin=  1.375 (R1//R2// Z(CGS))

Common Mistake 1: The circuit said vOUT, but this question asked for vout/vin. So the vout is supposed to be small signal output only, without the DC bias.

Common Mistake 2: Wrong small signal circuit. Some assumed CGS is infinite and shorted CGS in the small signal circuit.

BONUS [2 points]: Find the output impedance of this configuration.

Rout = 1gm//ro+Rs

Rout=ro+Rs+gm⋅Rs⋅ro