The Term Project of UC Berkeley CS 150
Components and Design Techniques for Digital Systems
Audio Broadcasting Over Ethernet
Filtering Network Data into Audio Data

[image: image6.png]FullVolumeControl.v

RegMuteMaster
~imute master { ) Sfecer Clock [audioclock
VolMaster [21 Enable  In
— Set out
imute master
UDCounterMaster
ivol master In
IO AP o Y B
[1] Count 5b
51 Down 5
Load

[ —
AudioClock =

RegMuteLineout.
[Reset Clock |<audioclock

VolLineout —>| 4 TZT SlEnable  In
‘ set out

~imute_lineout

imute_lineout
UDCounterLineQut|
ivol lineout >[In N SN Y
0 I 1b| ¢ 5 501
Lo Down Count| >[50 5 23
5 s '
151 > Stoad 16 mcqq;i 16
167h0001
Audiotlock 3|
RegMuteWave
$HLW [Reset Clock | audioclock o fe7Tn02
VolWave » Enable  In .
— Set out 1[STn0
Imute_wave bt 2 [€7n1s
3290
UDCounterWave 7 It 2;,2%\
ivol wave >[In
e m— N N
Count
5 Down K ThreeBitCounter
PN ] Load Enable count%
AudioClock 3| — Reset Clock
CMD ATn 7__CMD_AQut j
- CMD_OutRequest AudioClock
CMD_InRequest: ZudToReset [ f
AudioReset 9D Dout ==3"b100
CMD_DIn 1 —

AudioClock —thudioClock





Eth2Data.v

	Signal
	Width
	Dir
	Description

	DIn
	32
	I
	Packet data input bus

	InValid
	1
	I
	Indicates that DIn is valid on this cycle

	InPacketValid
	1
	I
	Indicates the end of a packet with a valid CRC

	InPacketInvalid
	1
	I
	Indicates the end of a packet with an invalid CRC

	EthernetClock
	1
	I
	25MHz Ethernet Clock

	EthernetReset
	1
	I
	EthernetClock synchronous reset

	OutClock
	1
	I
	12.288MHz Audio Clock

	OutReset
	1
	I
	OutClock synchronous reset

	DOut
	32
	O
	Audio data output bus

	OutRequest
	1
	I
	Request for a new audio word output

	OutValid
	1
	O
	Indicates that DOut is currently valid

	datawords
	 
	P
	There should be a datawords parameter, which will allow you to easily change the size of the packet decoded by this module


Audio Volume Control


[image: image2]




AC97Controller.v

	Signal
	Width
	Dir
	Description

	AP_SDATA_OUT
	1
	O
	AC97 serial data output to the LM4549A

	AP_BIT_CLOCK
	1
	I
	A 12.288MHz gated clock.  You should use this for all audio circuitry.

	AP_SDATA_IN
	1
	I
	AC97 serial data input from the LM4549A

	AP_SYNC
	1
	O
	AC97 frame synchronization signal

	AP_RESET_
	1
	O
	Active low (Notice the _) reset for the LM4549A.  This will gate the AP_BIT_CLOCK

	AP_PC_BEEP
	1
	O
	PC beep output.  Tie to 1’b0.

	Reset
	1
	I
	System reset, synchronous to Clock, used with LocalResetGen

	Clock
	1
	I
	System 27MHz free running clock, used with LocalResetGen

	PCM_DIn
	32
	I
	32bit PCM data input.  Bits [31:16] is the left sample, bits [15:0] is the right sample.

	PCM_InValid
	1
	I
	Data valid input.  Indicates that PCM_DIn is valid.  Driven by Eth2Data.

	PCM_InRequest
	1
	O
	PCM_DIn request.  Causes a read from the Eth2Data FIFO.

	PCM_DOut
	32
	O
	PCM data output.  This would carry the data read from the microphone.  Unused this semester.

	PCM_OutValid
	1
	O
	Indicates that PCM_DOut has valid data.  Unused this semester.

	CMD_AIn
	7
	I
	The address of the AC97 register to write.

	CMD_DIn
	16
	I
	Data to write to the AC97 register indicated by CMD_AIn.

	CMD_InValid
	1
	I
	Indicates that CMD_DIn and CMD_AIn are valid in response to CMD_InRequest.

	CMD_InRequest
	1
	O
	Request by this controller for the next AC97 register to write.

	AudioReset
	1
	O
	Audio system reset.  Generated  by LocalResetGen.

	AudioClock
	1
	O
	12.288MHz audio system clock.  Another name for AP_BIT_CLOCK.


FullVolumeControl.v

	Signal
	Width
	Dir
	Description

	VolMaster
	6
	I
	Volume control for all audio being played out the headphone jack on the CaLinx2 board.  Notice, there is a blue potentiometer which affects this also.

	VolLineOut
	6
	I
	Volume control for all audio being played out the RCA Line Out Jacks on the CaLinx2 board.

	VolLineIn
	6
	I
	Unused this semester.

	VolWave
	6
	I
	Volume control for all PCM data from the FPGA.

	VolMic
	6
	I
	Unused this semester

	VolRecordGain
	6
	I
	Unused this semester

	RecordSource
	3
	I
	Unused this semester

	CMD_AOut
	7
	O
	The address of the AC97 register to write.

	CMD_DOut
	16
	O
	Data to write to the AC97 register indicated by CMD_AOut.

	CMD_OutValid
	1
	O
	Indicates that CMD_DOut and CMD_AOut are valid in response to CMD_OutRequest.

	CMD_OutRequest
	1
	I
	Request from AC97Controller for the next AC97 register to write.

	AudioReset
	1
	I
	Audio system reset.

	AudioClock
	1
	I
	12.288MHz audio system clock.  Another name for AP_BIT_CLOCK.


[image: image1][image: image4.png]EE 150 Term Project Audio Network Block Diagram
Eth2Datax
OutClock
InPacketValid |
InPacketlnvalid pu
EthernetReset £
Reset
Counter
Enghle Cockl«—
FIFO 7" Dout
l X piv v
[Clock Set] S rd clleainit ful g |
ot s & R .
= - Set =N
k) e > OutValid
Din Reset — wr_clk Rija—rz/ utvall
SourceSelect —t 7N rd_ack
Reg2Reset rd_en
Counter >3
fnvalid Counter=0or1or2or3 ounter >
EthernetClock
OutRequest >
OutReset >/




[image: image5.png]AC97Controller. v

LocalReserGen.v

udioReser
LocalRegReser| >

cloen —
ese reser Localciockreses ¢
saioss 3o
b
s
bou
] —
nzvassa s —
o euest Coue-L [F 3 [comezs o
e LRequestRegut P, L C il 3 OMD_a0ut
[ ReeduestRegr ReadyReg| RightReqest Reg ap_bour
mssion s 2o (] o
: | +
EEe
ot [
e oo | o | FOE | B Jions - comseass
Ao L ter | @R | BB | G| HEmamande 1w
X w | B 5
o > andioReser
e
B Ty )
¥ tsoticar
ol comeezss or Comsd s 19 w JateC1ac

Comed ©
BBEg s ot
Reger




